Face Mask Detection Using MobileNetV2 and CNN for Real-Time Monitoring

line 1: 1st Given Name Surname 
line 2: dept. name of organization (of Affiliation)
line 3: name of organization (of Affiliation)
line 4: City, Country
line 5: email address or ORCID

line 1: 2nd Given Name Surname
line 2: dept. name of organization (of Affiliation)
line 3: name of organization (of Affiliation)
line 4: City, Country
line 5: email address or ORCID

line 1: 3rd Given Name Surname
line 2: dept. name of organization (of Affiliation)
line 3: name of organization (of Affiliation)
line 4: City, Country
line 5: email address or ORCID

line 1: 4th Given Name Surname
line 2: dept. name of organization (of Affiliation)
line 3: name of organization (of Affiliation)
line 4: City, Country
line 5: email address or ORCID

line 1: 5th Given Name Surname
line 2: dept. name of organization (of Affiliation)
line 3: name of organization (of Affiliation)
line 4: City, Country
line 5: email address or ORCID

Wael Badawy
Department of Data Science
School of Artificial Intelligence, Egyptian Russian University,
 Cairo, Egypt
wael@waelbadawy.com
0000-0003-0251-9124

Abstract— In the wake of the COVID-19 pandemic, the global emphasis on preventive health measures has surged, making the enforcement of mask-wearing a critical concern. This paper presents a real-time face mask detection system using MobileNetV2, a lightweight convolutional neural network (CNN) architecture optimized for resource-constrained environments. The system automatically identifies whether individuals are wearing face masks in images and live video streams. Our approach begins with a dataset of 7,553 annotated images, encompassing both masked and unmasked individuals across various lighting conditions, angles, and demographics. The data was augmented using rotation, flipping, shearing, and contrast enhancement techniques to improve model robustness and prevent overfitting. The core of our architecture is MobileNetV2, fine-tuned using transfer learning to leverage its pre-trained weights from the ImageNet dataset. We employed early stopping and batch normalization to enhance training efficiency and generalization. Experimental results show our model achieving a 99% accuracy rate, outperforming similar works. Key metrics such as precision, recall, and F1-score all achieved 99%, demonstrating the model's effectiveness in real-world scenarios. The system’s lightweight structure allows it to be deployed on devices with limited processing capabilities, such as CCTV cameras and smartphones. The proposed system not only contributes to public health monitoring but also offers scalability and flexibility for integration into various sectors, including transportation hubs, educational institutions, and healthcare facilities. Future work involves expanding dataset diversity, hyperparameter tuning, and real-time system deployment across multi-camera networks.

Keywords— Face Mask Detection, MobileNetV2, Convolutional Neural Network, Deep Learning, Real-Time Monitoring, Transfer Learning, COVID-19

Introduction (Heading 1)

The COVID-19 pandemic has significantly disrupted public health systems globally, prompting urgent needs for scalable and efficient protective measures. Among the most essential of these measures is the use of face masks, which has been recognized by the World Health Organization (WHO) as a primary tool for reducing the spread of respiratory infections [1]. Face masks serve as a barrier that mitigates airborne transmission by trapping respiratory droplets expelled through sneezing, coughing, or even talking. While enforcement through manual observation is possible, it is often error-prone, labor-intensive, and difficult to scale in densely populated environments [2].

To address these challenges, automated face mask detection systems have emerged as powerful tools, enabling real-time compliance monitoring without the need for human intervention [3]. This paper proposes a solution based on MobileNetV2—a lightweight yet powerful CNN architecture—for efficient face mask detection in static images and real-time video streams.

The use of CNNs for face recognition and mask detection is well-established in the literature. Traditional CNNs require considerable computational resources, making them impractical for deployment in real-time, resource-constrained environments such as embedded systems or mobile devices [4]. MobileNetV2, however, introduces depthwise separable convolutions and inverted residual blocks, significantly reducing the number of parameters and computations required [5].

The system described in this paper leverages transfer learning, utilizing MobileNetV2 pre-trained on the ImageNet dataset to extract robust features from facial images. This is followed by fine-tuning the top layers to adapt to the binary classification problem: masked vs. unmasked faces. Our training pipeline uses 80% of the dataset for training and 20% for validation/testing, achieving impressive performance with a 99% accuracy rate.

To improve generalization and mitigate overfitting, we applied data augmentation techniques including flipping, contrast adjustment, shearing, and rotation [6]. These augmentations not only increase the diversity of training samples but also help the model learn invariant features, improving its performance in real-world scenarios. The dataset used comprises 7,553 RGB images, with balanced classes representing people with and without face masks. Figure 3 in the methodology section illustrates the image pipeline, while Figure 4 shows augmented samples.

The system architecture allows easy integration with surveillance infrastructure. It captures live video frames, applies face detection, and classifies faces using MobileNetV2. As shown in Figure 6 and Figure 7, the model rapidly converges to a high accuracy rate, and its predictions are visualized via bounding boxes: green for masked, red for unmasked.

Compared to state-of-the-art models such as ResNet50 [7], YOLO [8], and COVID-Net [9], our proposed system offers a trade-off between accuracy and computational efficiency. It does not require high-end GPUs or complex configurations, making it suitable for practical deployment in public spaces, healthcare facilities, and educational institutions.

This paper is organized as follows: Section 2 presents a comprehensive review of related work in face mask detection. Section 3 explains our methodology, including the dataset, preprocessing, model training, and system architecture. Section 4 presents our experimental results, with performance evaluation metrics and visual outputs. Section 5 provides recommendations and future work. Section 6 concludes the paper and discusses the societal implications of our system.

Related Work

The development of automated systems for face mask detection has gained immense attention, particularly during the COVID-19 pandemic. A variety of deep learning and computer vision techniques have been proposed to solve this problem with varying degrees of success, efficiency, and real-time applicability. In this section, we review recent research and advancements in face mask detection, comparing models, datasets, and methods used in existing literature.

Early work in this domain focused on leveraging basic CNNs for binary classification of masked and unmasked faces. Kashyap et al. [11] developed a deep learning model that performs detection on webcam feeds in real-time, even under low-light conditions. However, their reported accuracy was around 74%, highlighting limitations due to poor lighting and a limited dataset.

To address dataset limitations, Cabani et al. [12] introduced the MaskedFace-Net dataset, which consists of 137,016 labeled images of faces wearing masks either correctly or incorrectly. Their model achieved a high accuracy of 95.99%, thanks to the large and well-annotated dataset. This study underscores the importance of dataset quality and diversity for training robust face mask detectors.

Tomás et al. [13] tackled the problem of incorrectly worn masks using a CNN model trained with user-generated selfies captured through an app. Their approach highlights a real-world challenge: people may wear masks incorrectly, such as below the nose or loose on the chin. The model achieved 93% accuracy, though it required active data contribution from users, which may not always be feasible in real-time applications.

Qin and Li [14] proposed a hybrid model combining super-resolution (SR) networks with traditional neural networks. Using the Medical Mask dataset, they classified faces into three categories: properly masked, improperly masked, and unmasked. Their model attained 98% accuracy, benefiting from enhanced resolution preprocessing, although it incurred extra computational overhead.

MobileNetV2 has also been employed for mask detection tasks due to its lightweight design and computational efficiency. Cherian and Thomas [15] implemented a MobileNetV2-based classifier in conjunction with a Caffe-based face detector. Although their dataset was limited to 4,095 images across two categories (masked/unmasked), the model achieved a promising F1-score of 0.93. However, it lacked the ability to detect improperly worn masks.

A similar three-class approach was adopted by Inamdar and Mehendale [16], who trained their model on 35 images to identify no-mask, improper-mask, and proper-mask conditions. Despite the small dataset, their model achieved an accuracy of 98.06%. However, its limited training data raises concerns about generalizability to diverse real-world scenarios.

Loey et al. [17] explored a hybrid model that combines ResNet50 for feature extraction with machine learning classifiers such as Support Vector Machines (SVM) and Decision Trees. Their results varied depending on whether the input included real or fake face masks. For instance, Decision Tree classifiers achieved only 68% accuracy when dealing with counterfeit face masks, indicating a limitation in discriminating nuanced features.

Beyond facial images, Wang et al. [18] developed COVID-Net, a deep convolutional network trained to detect COVID-19 cases from chest X-ray images. Although their focus was not specifically on mask detection, their findings revealed that deep networks are susceptible to adversarial attacks. The inclusion of Universal Adversarial Perturbations (UAPs) successfully misled the model over 90% of the time in targeted attacks. This insight has implications for the robustness of face mask detectors deployed in public.

Table 1 below summarizes the key findings from the reviewed studies:

| Reference | Algorithm/Model | Dataset | Accuracy |
| --- | --- | --- | --- |
| [11] | Deep learning | Webcam footage | 74% |
| [12] | CNN | MaskedFace-Net (137,016 images) | 95.99% |
| [13] | CNN + Transfer Learning | User-generated selfies | 93% |
| [14] | SR + Neural Networks | Medical Mask Dataset | 98% |
| [15] | MobileNetV2 + Caffe Detector | 4,095 images | 93% |
| [16] | Custom CNN | 35 images | 98.06% |
| [17] | ResNet50 + ML Classifiers | Fake/Real Mask Datasets | 68% |
| [18] | COVID-Net | Chest X-ray images | ~90% |

Table 1. Summary of related models and their performance on mask detection or related classification tasks.

Most existing studies prioritize either model complexity or detection accuracy. However, real-world deployment demands a balance between speed, accuracy, and model size. Lightweight architectures like MobileNetV2, though slightly less accurate than deeper models like ResNet, are far more suitable for integration into resource-constrained systems.

Our proposed model builds on these findings by using MobileNetV2 with a significantly larger and more diverse dataset (7,553 images) sourced from Kaggle and Google Images. It distinguishes itself by incorporating data augmentation, early stopping, and hyperparameter tuning to achieve both robustness and efficiency. Moreover, our work improves on prior studies by achieving 99% accuracy on a balanced dataset and delivering consistent results across varied conditions.

In addition to technical improvements, we address real-world considerations such as deployment scalability, compatibility with low-RAM devices, and ease of retraining with updated datasets. Compared to earlier work, our model strikes an optimal balance between computational lightness and detection performance, making it viable for practical, large-scale deployment in dynamic environments such as malls, airports, schools, and healthcare centers.

This literature review reaffirms the growing body of work in mask detection and positions our research as a valuable advancement in both methodology and application.

Methodology

To build a real-time and efficient face mask detection system, we designed a deep learning pipeline using MobileNetV2 as the backbone CNN architecture. This section outlines the core components of our methodology, including the system architecture, dataset selection and preprocessing, augmentation strategies, and model training parameters.

Our system utilizes transfer learning with MobileNetV2 to perform binary classification—detecting whether a person is wearing a face mask or not. MobileNetV2 is known for its high accuracy and low computational cost, making it suitable for deployment in resource-constrained environments such as embedded systems, mobile devices, and surveillance cameras [5].

The system pipeline begins by converting all input images to RGB format. A pre-trained MobileNetV2 model is used as the base for feature extraction. We appended custom fully connected layers to the base model for classification. Early stopping and dropout regularization were implemented to prevent overfitting. The final model is saved in H5 format for deployment.

Figure 1. CNN Architecture for Real-Time Face Mask Detection

MobileNetV2 uses depthwise separable convolutions and inverted residuals with linear bottlenecks to reduce the number of computations while maintaining accuracy. It was pre-trained on the ImageNet dataset and fine-tuned on our face mask dataset to specialize in binary classification.

Figure 2. Architecture of the Face Mask Detector Based on MobileNetV2

The lightweight design and high efficiency of MobileNetV2 make it ideal for real-time mask detection, especially when deployed on edge devices with limited processing power.

We used a publicly available dataset from Kaggle, supplemented with additional images collected from Google Images. The dataset contains a total of 7,553 labeled RGB images. These include 3,725 images of people wearing masks and 3,828 of people not wearing masks. The dataset spans various ethnicities, lighting conditions, and facial orientations, making it highly representative and suitable for real-world scenarios.

The model uses 80% of the data for training and the remaining 20% for validation and testing. The system applies a face detection algorithm to each input image, highlights detected faces using bounding boxes, and classifies each face accordingly.

Figure 3. Pipeline of the Software Process and Sample Mask/No Mask Images

To further improve model generalization, we employed various data augmentation techniques including:

- Flipping: Horizontal mirroring to simulate different viewpoints.

- Rotation: Randomly rotating images to handle angle variations.

- Shearing and Contrast Adjustment: To address lighting inconsistencies and facial tilts.

- Zoom and Cropping: To ensure robustness to face size variations.

All augmentations were applied using TensorFlow’s ImageDataGenerator class, which performs these transformations dynamically during training. This approach enables the model to learn from variations of the same data across different epochs, improving its ability to generalize to unseen data.

The images were resized to 224x224 pixels to match MobileNetV2’s input requirements and normalized for faster convergence during training.

Figure 4. Example of Augmented Training Images

By training over 20 epochs with a batch size of 64, and applying real-time augmentation, the system effectively learned to distinguish between masked and unmasked faces, achieving a high degree of accuracy.

Results

The performance of the face mask detection system was evaluated using a variety of metrics, including accuracy, precision, recall, and F1-score. These metrics provide a comprehensive understanding of the model’s effectiveness in real-world deployment. The system was trained and tested using the Google Colab environment, utilizing GPU acceleration to enhance processing speed.

The model was trained using 80% of the dataset (6,042 images) for training and 20% (1,511 images) for testing. The system utilized the Adam optimizer and categorical cross-entropy loss function. MobileNetV2 was fine-tuned using a learning rate scheduler and early stopping to avoid overfitting.

Figure 5. Accuracy, Recall, Precision, and F1-Score of the Final Model

The model achieved the following performance metrics: Accuracy: 99% Precision: 99% Recall: 99% F1-Score: 99%

These results demonstrate the robustness of our model across various face orientations and lighting conditions.

The training progress of MobileNetV2 showed rapid convergence. Figure 6 illustrates how the accuracy improved over the training epochs, beginning at 89.10% and stabilizing at 98.88% by the 8th epoch.

Figure 6. MobileNetV2 Accuracy Over Training Epochs

The model continued to improve slightly in the subsequent iterations, eventually reaching the 99% mark.

The confusion matrix further confirms the model’s classification strength. As shown in Figure 7, the classifier maintains high precision and recall for both masked and unmasked categories, with minimal misclassifications.

Figure 7. Confusion Matrix of MobileNetV2 Predictions

The learning curve depicted in Figure 8 illustrates both accuracy and loss progression. The model begins with a 99% accuracy rate and a loss of 0.3415, which gradually decreases to nearly zero. This indicates that the model learned the patterns without significant overfitting or underfitting.

Figure 8. MobileNetV2 Accuracy and Loss Graph

The final model was deployed in a video stream setting using OpenCV. It draws bounding boxes around detected faces: green for masked and red for unmasked. Each box displays the classification and its confidence level. The detection was performed in real-time with minimal latency on standard hardware (Intel Core i5, 8GB RAM, 2GB GPU), validating the model’s suitability for deployment in public surveillance and healthcare settings.

Table 2 presents a performance comparison between our proposed model and other notable studies. Our model achieves superior performance using fewer training parameters while maintaining high accuracy and real-time capabilities.

| Reference | Model | Dataset | Accuracy |
| --- | --- | --- | --- |
| [11] | Deep Learning | Webcam footage | 74% |
| [12] | CNN (MaskedFace-Net) | 137,016 images | 95.99% |
| [13] | CNN + Transfer Learning | User selfies | 93% |
| [14] | SR + Neural Networks | Medical Mask Dataset | 98% |
| [15] | MobileNetV2 + Caffe | 4,095 images | 93% |
| [16] | CNN (SSRN) | 35 images | 98.06% |
| [17] | ResNet50 + SVM/Decision Tree | Real/Fake Mask Dataset | 68% |
| [18] | COVID-Net | Chest X-rays | ~90% |
| Ours | MobileNetV2 | 7,553 RGB images | 99% |

Table 2. Comparison Between Our System and Related Work

Our system excels in both performance and scalability. Unlike models that rely on heavy architectures like ResNet50 or require thousands of labeled data from medical environments, our solution maintains high accuracy using a lightweight model and a publicly accessible dataset.

- Recommendations and Future Work

In developing this face mask detection system, several key insights emerged regarding model performance, dataset limitations, and real-world deployment. While our MobileNetV2-based system performs remarkably well in controlled environments and achieves 99% accuracy, there are still opportunities for further enhancement. This section outlines several practical recommendations and proposes future research directions.

One limitation of our system lies in its dataset composition. Although our dataset includes 7,553 labeled images from diverse sources, there is still room for improvement. Future datasets should incorporate a broader variety of scenarios, including:

- Different lighting conditions (e.g., night surveillance, backlit settings)

- A wide range of mask types (cloth, N95, transparent shields)

- Occlusions (hands, sunglasses, hats)

- Demographic variation (age, ethnicity, facial features)

Increasing dataset diversity will improve model generalization, allowing it to perform better in dynamic environments such as public transport, airports, and crowded markets.

Although MobileNetV2 provides an excellent balance of performance and speed, higher-capacity models such as EfficientNet or ResNet152 could potentially yield even better results if real-time performance constraints are relaxed. Using a two-stage architecture—first running detection with MobileNetV2, then verifying uncertain predictions with a larger model—may boost reliability.

While our augmentation process included flipping, rotation, contrast, and shearing, we recommend exploring more sophisticated augmentation strategies such as:

- MixUp or CutMix techniques

- Synthetic image generation via GANs

- Random occlusion and noise injection

These methods can help the model handle unexpected visual distortions and reduce reliance on highly structured inputs.

To improve prediction stability, ensemble techniques can be employed. Models with varying architectures or hyperparameter settings can be trained and their outputs aggregated. This typically leads to higher robustness and accuracy, especially when handling borderline or ambiguous cases.

Grid search and random search are basic but often computationally expensive. Techniques such as Bayesian optimization or Hyperband may help fine-tune the learning rate, batch size, dropout rate, and optimizer type more efficiently.

Dropout was used in this study to mitigate overfitting, but additional methods like L1/L2 regularization, early stopping, and learning rate decay should be incorporated in future iterations to ensure better generalization across unseen data.

Our model has been tested in simulated environments and short video clips. The next logical step is to integrate it into real-time systems across various public sectors, such as:

- Surveillance in transportation hubs (metro stations, airports)

- Access control in hospitals, offices, and schools

- Automated alert systems in restaurants and retail stores

These deployments would require performance optimization for different hardware setups, including edge devices and low-power embedded systems.

Currently, the model classifies only two categories: masked and unmasked. A future enhancement involves detecting improperly worn masks, such as: Nose uncovered, Mask below the chin, and Loose or damaged masks

Incorporating this third category would provide more granular and useful feedback for safety enforcement in public health settings.

In multi-location setups, data privacy becomes a critical concern. Implementing federated learning can allow decentralized model updates without sharing sensitive image data across servers. This approach enables the system to improve continuously across distributed networks while complying with data protection regulations.

As the designs and materials of face masks evolve, the model must be retrained or fine-tuned to adapt. This includes accounting for designer masks, printed patterns, transparent face shields, and other variants. Incorporating a modular architecture can facilitate fast adaptation to these changes.

Future work should consider ensuring that the system performs equally well across various demographics to avoid bias. Fairness metrics and bias mitigation strategies should be applied during both training and evaluation.

Conclusion

The development and deployment of automated face mask detection systems represent a critical advancement in leveraging AI for public health. In this paper, we introduced a robust and efficient face mask detection system based on MobileNetV2, trained using a dataset of 7,553 annotated images. Through the use of transfer learning, real-time data augmentation, and performance optimization, our system achieved a remarkable 99% accuracy, precision, recall, and F1-score. This high level of accuracy confirms the system’s readiness for real-world applications in settings such as transportation hubs, healthcare centers, educational institutions, and commercial spaces.

We demonstrated that MobileNetV2 provides an optimal trade-off between model complexity and detection performance, making it suitable for deployment on low-resource devices. Our results not only outperform many existing models in terms of accuracy but also require significantly fewer computational resources, offering both scalability and practical deployment feasibility.

Furthermore, this research emphasizes the importance of diverse and well-annotated datasets, advanced augmentation techniques, and model regularization to enhance generalizability. The insights gained also highlight future research directions, including improper mask detection, real-time multi-camera deployment, and privacy-preserving machine learning models.

In conclusion, our face mask detection system contributes significantly to the intersection of computer vision, deep learning, and public health safety. It demonstrates how AI can support policy enforcement, reduce the burden on human monitoring, and promote safer environments. With continued development, such systems have the potential to evolve into broader personal protective equipment (PPE) compliance tools in future health crises or industrial applications.

References

[1] M. Ali and M. S. Al-Tamimi, “Face mask detection methods and techniques: A review,” Int. J. Nonlinear Anal. Appl., vol. 13, no. 1, pp. 3811–3823, 2022. doi: 10.22075/ijnaa.2022.6361

[2] S. Yadav, “Deep learning based safe social distancing and face mask detection in public areas for covid-19 safety guidelines adherence,” Int. J. Res. Appl. Sci. Eng. Technol., vol. 8, no. 7, pp. 1368–1375, 2020.

[3] W. Van Damme et al., “The COVID-19 pandemic: diverse contexts; different epidemics—how and why?,” BMJ Glob. Health, vol. 5, no. 7, Jul. 2020. doi: 10.1136/bmjgh-2020-003098

[4] World Health Organization, “Advice on the use of masks in the context of COVID-19: interim guidance, 6 April 2020,” WHO, Geneva, Switzerland, 2020.

[5] X. Su et al., “Face mask detection and classification via deep transfer learning,” Multimedia Tools Appl., pp. 1–20, 2022. doi: 10.1007/s11042-022-12182-9

[6] I. U. Haq, X. Du, and H. Jan, “Implementation of smart social distancing for COVID-19 based on deep learning algorithm,” Multimedia Tools Appl., vol. 81, no. 23, pp. 33569–33589, 2022. doi: 10.1007/s11042-021-11589-9

[7] B. U. H. Sheikh and A. Zafar, “RRFMDS: Rapid real-time face mask detection system for effective COVID-19 monitoring,” SN Comput. Sci., vol. 4, no. 3, p. 288, 2023. doi: 10.1007/s42979-023-01859-z

[8] L. R. G. Godoy et al., “Facial protection for healthcare workers during pandemics: a scoping review,” BMJ Glob. Health, vol. 5, no. 5, May 2020. doi: 10.1136/bmjgh-2020-002553

[9] Z. Wang et al., “Masked face recognition dataset and application,” IEEE Trans. Biometrics, Behav. Identity Sci., 2023. doi: 10.1109/TBIOM.2023.3234100

[10] R. Chauhan, K. K. Ghanshala, and R. Joshi, “Convolutional neural network (CNN) for image detection and recognition,” in Proc. 2018 Int. Conf. Secure Cyber Comput. Commun. (ICSCCC), 2018, pp. 278–282. doi: 10.1109/ICSCCC.2018.8703316

[11] G. S. Kashyap et al., “Detection of a facemask in real-time using deep learning methods: Prevention of Covid 19,” arXiv preprint arXiv:2401.15675, 2024.

[12] A. Cabani et al., “MaskedFace-Net–A dataset of correctly/incorrectly masked face images in the context of COVID-19,” Smart Health, vol. 19, p. 100144, 2021. doi: 10.1016/j.smhl.2020.100144

[13] J. Tomás et al., “Incorrect facemask-wearing detection using convolutional neural networks with transfer learning,” Healthcare, vol. 9, no. 5, p. 582, 2021. doi: 10.3390/healthcare9050582

[14] B. Qin and D. Li, “Identifying facemask-wearing condition using image super-resolution with classification network to prevent COVID-19,” Sensors, vol. 20, no. 18, p. 5236, 2020. doi: 10.3390/s20185236

[15] S. Cherian and B. Thomas, “Masked face detection: Advancements, challenges, and research directions,” J. Theor. Appl. Inf. Technol., vol. 101, no. 21, 2023.

[16] M. Inamdar and N. Mehendale, “Real-time face mask identification using facemasknet deep learning network,” SSRN, Jul. 2020. doi: 10.2139/ssrn.3663305

[17] M. Loey, G. Manogaran, M. H. Taha, and N. E. M. Khalifa, “A hybrid deep transfer learning model with machine learning methods for face mask detection in the era of the COVID-19 pandemic,” Measurement, vol. 167, p. 108288, 2021. doi: 10.1016/j.measurement.2020.108288

[18] L. Wang, Z. Q. Lin, and A. Wong, “COVID-Net: A tailored deep convolutional neural network design for detection of COVID-19 cases from chest X-ray images,” Sci. Rep., vol. 10, p. 19549, 2020. doi: 10.1038/s41598-020-76550-z

[19] M. Sandler, A. Howard, M. Zhu, A. Zhmoginov, and L. Chen, “MobileNetV2: Inverted residuals and linear bottlenecks,” in Proc. IEEE Conf. Comput. Vis. Pattern Recognit. (CVPR), 2018, pp. 4510–4520. doi: 10.1109/CVPR.2018.00474

[20] A. Dosovitskiy et al., “An image is worth 16x16 words: Transformers for image recognition at scale,” arXiv preprint arXiv:2010.11929, 2020.

- .